# The Extended Concise LCD Data Sheet	for HD44780

Version: 25.6.1999

| Instruction | RS | RW | D7 | D6 |  | D5 | D4 | D3 | D2 | D1 |  | D0 |  | Description |  |  | Clock- Cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NOP | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |  | 0 |  | No Operation |  |  | 0 |
| Clear Display | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |  | 1 |  | Clear display & set address counter to zero |  |  | 165 |
| Cursor Home | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 1 |  | x |  | Set adress counter to zero, return shifted display to original position. DD RAM contents remains unchanged. |  |  | 3 |
| Entry Mode Set | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 1 | I/D |  | S |  | Set cursor move direction (I/D) and specify automatic display shift (S). |  |  | 3 |
| Display Control | 0 | 0 | 0 | 0 |  | 0 | 0 | 1 | D | C |  | B |  | Turn display (D), cursor on/off (C), and cursor blinking (B). |  |  | 3 |
| Cursor / Display shift | 0 | 0 | 0 | 0 |  | 0 | 1 | S/C | R/L | x |  | x |  | Shift display or move cursor (S/C) and specify direction (R/L). |  |  | 3 |
| Function Set | 0 | 0 | 0 | 0 |  | 1 | DL | N | F | x |  | x |  | Set interface data width (DL), number of display lines (N) and character font (F). |  |  | 3 |
| Set CGRAM Address | 0 | 0 | 0 | 1 | CGRAM Address |  |  |  |  |  |  |  |  |  | Set CGRAM address. CGRAM data is sent afterwards. |  | 3 |
| Set DDRAM Address | 0 | 0 | 1 | DDRAM Address |  |  |  |  |  |  |  |  |  |  | Set DDRAM address. DDRAM data is sent afterwards. |  | 3 |
| Busy Flag & Address | 0 | 1 | BF | Address Counter |  |  |  |  |  |  |  |  |  |  | Read busy flag (BF) and address counter |  | 0 |
| Write Data | 1 | 0 | Data |  |  |  |  |  |  |  |  |  |  |  | Write data into DDRAM or CGRAM |  | 3 |
| Read Data | 1 | 1 | Data |  |  |  |  |  |  |  |  |  |  |  | Read data from DDRAM or CGRAM |  | 3 |
| x : Don't care | I/D | 1 0 | Increment Decrement |  |  |  |  |  |  |  | R/L |  | 1 0 |  |  | Shift to the right Shift to the left |  |
|  | S | 1 0 | Automatic display shift |  |  |  |  |  |  |  | DL |  | 1 0 |  |  | 8 bit interface 4 bit interface |  |
|  | D | 1 0 | Display ON Display OFF |  |  |  |  |  |  |  | N |  | 1 0 |  |  | 2 lines 1 line |  |
|  | C | 1 0 | Cursor ON Cursor OFF |  |  |  |  |  |  |  | F |  | 1 0 |  |  | 5x10 dots 5x7 dots |  |
|  | B | 1 0 | Cursor blinking |  |  |  |  |  |  |  | DDRAM : Display Data RAM CGRAM : Character Generator RAM |  |  |  |  |  |  |
|  | S/C | 1 0 | Display shift Cursor move |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

LCD Display with 2 lines x 40 characters :

LCD Display with 2 lines x 16 characters :

| Pin No | Name | Function | Description |
| --- | --- | --- | --- |
| 1 | Vss | Power | GND |
| 2 | Vdd | Power | + 5 V |
| 3 | Vee | Contrast Adj. | (-2) 0 - 5 V |
| 4 | RS | Command | Register Select |
| 5 | R/W | Command | Read / Write |
| 6 | E | Command | Enable (Strobe) |
| 7 | D0 | I/O | Data LSB |
| 8 | D1 | I/O | Data |
| 9 | D2 | I/O | Data |
| 10 | D3 | I/O | Data |
| 11 | D4 | I/O | Data |
| 12 | D5 | I/O | Data |
| 13 | D6 | I/O | Data |
| 14 | D7 | I/O | Data MSB |

Bus Timing Characteristics

( Ta = - 20 to + 75°C )

| Write-Cycle | VDD | 2.7 - 4.5 V (2) | 4.5 - 5.5 V (2) |  | 2.7 - 4.5 V (2) | 4.5 - 5.5 V (2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Min(1) |  | Typ(1) | Max(1) |  | Unit |
| Enable Cycle Time | tc | 1000 | 500 | - | - | - | ns |
| Enable Pulse Width (High) | tw | 450 | 230 | - | - | - | ns |
| Enable Rise/Fall Time | tr, tf | - | - | - | 25 | 20 | ns |
| Address Setup Time | tas | 60 | 40 | - | - | - | ns |
| Address Hold Time | tah | 20 | 10 | - | - | - | ns |
| Data Setup Time | tds | 195 | 80 | - | - | - | ns |
| Data Hold Time | th | 10 | 10 | - | - | - | ns |

- The above specifications are indications only (based on Hitachi HD44780). Timing will vary from manufacturer to manufacturer.

- Power Supply :	HD44780 S :	VDD = 4.5 - 5.5 V
 	HD44780 U :	VDD = 2.7 - 5.5 V

This data sheet refers to specifications for the Hitachi HD44780 LCD Driver chip, which is used for most LCD modules.

Common types are :	1 line x 20 characters
2 lines x 16 characters
2 lines x 20 characters
2 lines x 40 characters
4 lines x 20 characters
4 lines x 40 characters

© 1998/1999 by	Craig Peacock, Australia	http://www.beyondlogic.org
Peter Luethi, Switzerland	http://www.electronic-engineering.ch